LYX MotionPLC

Instruction Reference

V4.02 Task Programming and Debugging Guide

| Applicable Software | LYX MotionPLC V4.02 |
| --- | --- |
| Document Version | V1.0 |
| Release Date | 2026-07-30 |

Liyuxun Automation Technology

# Using This Reference and Safety Information

Safety Warning Motion, homing, pick/place, and output instructions directly drive mechanisms or external equipment. Before the first run, disconnect the load or reduce the speed, and verify limits, emergency stop, direction, points, and safe height.

This reference is based on the implementation of LYX MotionPLC V4.02 host software and firmware. It explains the 27 task instructions currently available in the user interface. Reserved instructions not exposed by the interface are not included.

Recommended reading order: review the execution model and parameter conventions first, then look up instructions by category. Values in the examples are illustrative only. Actual values must be set according to machine travel, mechanical direction, and process requirements.

## Contents

1 Task Execution Model and Debug Display

2 Parameter, IO, and Variable Conventions

3 Motion and Interpolation Instructions (15)

4 Vision Offset Instructions (2)

5 IO and Timing Instructions (2)

6 Variables and Flow Control (3)

7 Auxiliary Motors and Indexer (4)

8 End Instruction (1)

9 Complete Task Examples

10 Debugging, Pause, and Stop

Appendix A - Complete Instruction Quick Reference

# 1 Task Execution Model and Debug Display

LYX MotionPLC can store Tasks 0-9 and provides one Test slot. A task consists of multiple instruction rows. At runtime, the program counter starts at row 0. A normal instruction advances to the next row when complete, while a jump instruction can change the next row.

Blocking instructions: motion, delay, fixed-pulse motor, vision request, and pick/place instructions wait for the action to complete before advancing.

Immediate instructions: IO_WRITE, SET_VAR, JUMP, and continuous motor start normally advance immediately after performing their action.

Resource interlock: if another task owns a main-motion, auxiliary-motor, or Y-output resource, the current task waits instead of forcibly taking the resource.

Current-row display: V4.02 refreshes the active instruction row through the heartbeat at approximately 300 ms intervals. A valid row is shown only while a task is running or paused, and the row is highlighted or selected for debugging.

Restarting a task returns its execution position to row 0 and clears that task's temporary wait state. It does not automatically clear global variables or initialize all Y outputs.

Key Point END terminates the task execution flow; it does not perform system initialization. If a task requires initial variable values or output states, explicitly place SET_VAR and IO_WRITE instructions at the beginning of the task.

# 2 Parameter, IO, and Variable Conventions

| Object | Convention |
| --- | --- |
| Axes | X, Y, and Z are linear axes; R is a rotary axis. Positions are normally expressed in pulses. |
| Inputs | X0-X39, 40 points total. |
| Outputs | Y0-Y23, 24 points total. 0=Close/ON; 1=Open/OFF. |
| Variables | V0-V19, twenty 32-bit unsigned global variables. Task completion does not clear them automatically. |
| Points | The point table stores X/Y/Z/R. Some point parameters can use a literal point number or obtain the point number from a variable. |
| Time | Delay values are in milliseconds (ms). |
| Speed | Step pulse frequency is normally expressed in Hz; acceleration is expressed in steps/s2. |
| Vision ROI | ROI 1=Pick Offset; ROI 2=Place Offset. A vision offset is consumed after a compensation instruction reads it. |

Common IO Logic Error In the interface, Close is encoded as 0 and Open is encoded as 1. Do not interpret it using the common Boolean assumption that 1 means ON.

# 3 Motion and Interpolation Instructions

The 15 instructions in this section use the main-motion resource. An out-of-range target, invalid point, or incorrect axis configuration puts the task into an error state. If the resource is busy, the task waits.

### 1. Move to Point  -  MOVE_POINT_ID

Purpose: Move to the absolute X/Y/Z/R coordinates stored in the point table.

| Parameter | Description |
| --- | --- |
| point_index | Point number. Enter it directly or supply the point number through a variable. |

Execution: Resolve the point number -> read point XYZR -> check the range -> start absolute coordinated motion -> wait for completion.

Example  MOVE_POINT_ID  point_index=12

Note: The point must already be downloaded to the controller. A point number supplied by a variable must also be within the valid point range.

### 2. Move to XYZR  -  MOVE_XYZR

Purpose: Specify a four-axis absolute target directly.

| Parameter | Description |
| --- | --- |
| x / y / z | Absolute X, Y, and Z positions in pulses. |
| r | Absolute R-axis position or corresponding angle pulses. |

Execution: Check all four target ranges -> start absolute coordinated motion -> wait for completion -> advance.

Example  MOVE_XYZR  x=50000, y=12000, z=8000, r=0

Note: Use a low speed for initial debugging and confirm the direction of every axis.

### 3. Move to XYZ with Offset  -  MOVE_XYZ_WITH_OFFSET

Purpose: Apply a vision offset to a specified point and move to the resulting target.

| Parameter | Description |
| --- | --- |
| point_index | Base point number. |
| roi_id | 1=Pick Offset; 2=Place Offset. |
| offset_option | X, Y, or XY. |

Execution: Read the base point -> obtain the selected ROI offset -> apply X/Y/XY -> check the range -> move absolutely and wait.

Example  MOVE_XYZ_WITH_OFFSET  point=5, roi=1, option=XY

Note: First use REQUEST_ROI_OFFSET to obtain data for the corresponding ROI. The offset is consumed when read and cannot be assumed available again.

### 4. Relative Move  -  MOVE_RELATIVE

Purpose: Move X/Y/Z/R by signed increments from the current position.

| Parameter | Description |
| --- | --- |
| x / y / z / r | Signed relative increment for each axis; the sign determines direction. |

Execution: Current position + axis increments -> check the new target range -> start coordinated relative motion -> wait for completion.

Example  MOVE_RELATIVE  x=1000, y=-500, z=0, r=0

Note: Safety depends on the current position. Repeated use in a loop can accumulate into an out-of-range target.

### 5. Single-Axis Absolute Move  -  MOVE_AXIS_ABSOLUTE

Purpose: Move one axis to an absolute pulse position.

| Parameter | Description |
| --- | --- |
| axis | X, Y, Z, or R. |
| pulse_count | Absolute target pulse count; the interface requires a nonnegative value. |

Execution: Select the axis -> check the target -> start single-axis absolute motion -> wait for completion.

Example  MOVE_AXIS_ABSOLUTE  axis=Z, pulse_count=15000

Note: X/Y/Z are limited by their configured maximum travel. R is handled according to its rotary-axis configuration.

### 6. Single-Axis Relative Move  -  MOVE_AXIS_RELATIVE

Purpose: Move one axis by a specified pulse count relative to its current position.

| Parameter | Description |
| --- | --- |
| axis | X, Y, Z, or R. |
| pulse_count | Signed relative pulse count; may be positive or negative. |

Execution: Current position + relative pulses -> check the range -> start single-axis motion -> wait for completion.

Example  MOVE_AXIS_RELATIVE  axis=X, pulse_count=-2500

Note: A negative value moves in the reverse direction; it is not an absolute position.

### 7. XY Arc  -  ARC_XY

Purpose: Move along an arc in the XY plane while optionally reaching a target Z.

| Parameter | Description |
| --- | --- |
| x / y / z | Absolute coordinates of the arc endpoint. |
| i / j | X/Y offset from the current start point to the arc center. |
| direction | CW or CCW. |

Execution: Use the current position as the start -> calculate the center from I/J -> validate geometry and range -> interpolate the arc -> wait for completion.

Example  ARC_XY  x=10000, y=10000, z=0, i=0, j=10000, direction=CW

Note: I/J are center offsets relative to the start point, not absolute center coordinates.

### 8. XY Ellipse  -  ELLIPSE_XY

Purpose: Execute elliptical interpolation in the XY plane while optionally reaching a target Z.

| Parameter | Description |
| --- | --- |
| z | Endpoint Z. |
| i / j | Ellipse-center offset from the start point. |
| major_radius | Major-axis radius. |
| minor_radius | Minor-axis radius. |
| rotation_angle_x1000 | Major-axis direction angle x1000; for example, 45000=45 degrees. |
| direction | CW or CCW. |

Execution: Resolve the center, radii, and rotation angle -> validate parameters and range -> interpolate the ellipse -> wait for completion.

Example  ELLIPSE_XY  z=0, i=-1000, j=0, major=1000, minor=500, angle=0, CW

Note: The radii must match the intended path. The angle unit is 0.001 degree.

### 9. Pick and Place  -  PICK_PLACE

Purpose: Perform a complete pick-and-place sequence between two points.

| Parameter | Description |
| --- | --- |
| pick_point_index | Pick point number. |
| place_point_index | Place point number. |
| safe_z | Safe Z height used for transfers. |
| io_index | Vacuum or gripper Y output. |
| io_open_delay_ms | Wait after closing at the pick point. |
| io_close_delay_ms | Wait after opening at the place point. |

Execution: Rise to safe_z -> move to pick X and adjust pick R -> descend to pick Z -> close Y -> wait -> return to safe_z -> rotate to place R -> move to place XY -> descend to place Z -> open Y -> wait -> return to safe_z.

Example  PICK_PLACE  pick=1, place=2, safe_z=20000, Y3, 150 ms / 150 ms

Note: In the current firmware, the horizontal pick phase first moves to pick X while Y remains at its current position. Confirm during point design and machine debugging that this implemented path is safe.

### 10. Pick  -  PICK

Purpose: Perform a pick action at one point and return to the safe height.

| Parameter | Description |
| --- | --- |
| point_index | Pick point number. |
| safe_z | Safe Z height. |
| io_index | Y output. |
| io_value | Default 0=Close. |
| io_delay_ms | Wait after changing the output. |

Execution: Rise to safe_z -> move to point XY/R -> descend to point Z -> set Y -> wait -> return to safe_z.

Example  PICK  point=1, safe_z=20000, Y3=Close, delay=150

Note: Both the point Z and safe_z must be within the Z-axis range.

### 11. Place  -  PLACE

Purpose: Perform a place action at one point and return to the safe height.

| Parameter | Description |
| --- | --- |
| point_index | Place point number. |
| safe_z | Safe Z height. |
| io_index | Y output. |
| io_value | Default 1=Open. |
| io_delay_ms | Wait after changing the output. |

Execution: Rise to safe_z -> move to point XY/R -> descend to point Z -> set Y -> wait -> return to safe_z.

Example  PLACE  point=2, safe_z=20000, Y3=Open, delay=150

Note: The default place action opens the output. If the gripper logic is reversed, explicitly select the correct state.

### 12. Three-Stage Home  -  HOME

Purpose: Perform the complete three-stage homing sequence on the selected axes.

| Parameter | Description |
| --- | --- |
| axis_mask | Select X, Y, Z, R, a combination, or all axes. |

Execution: Home axes in firmware order: Z -> R when the R limit is enabled -> X -> Y. Advance after all selected axes finish.

Example  HOME  axis_mask=X+Y+Z+R

Note: Verify limit inputs and homing directions first. Selecting multiple axes does not mean simultaneous homing; the axes run in sequence.

### 13. Single-Stage Axis Reset  -  RESET_AXIS

Purpose: Perform a single-stage reset/homing operation on one axis.

| Parameter | Description |
| --- | --- |
| axis | X, Y, Z, or R. |

Execution: Call the selected axis's single-axis Reset workflow and wait for completion.

Example  RESET_AXIS  axis=Z

Note: This differs from the complete three-stage HOME sequence and is intended for a known single-stage reset requirement.

### 14. Set Speed and Acceleration  -  SET_SPEED_HZ

Purpose: Update linear and arc speed and acceleration parameters.

| Parameter | Description |
| --- | --- |
| speed_hz | Maximum linear speed. |
| accel_steps_s2 | Linear acceleration. |
| arc_speed_hz | Maximum arc speed. |
| arc_accel_steps_s2 | Arc acceleration. |

Execution: Validate each parameter. A zero value leaves the current setting unchanged; a nonzero value updates it. If start speed exceeds the new maximum, it is limited to the maximum.

Example  SET_SPEED_HZ  speed=8000, accel=20000, arc_speed=5000, arc_accel=12000

Note: Zero means do not modify; it does not set the speed to zero.

### 15. Homing Repeatability Test  -  TEST_PRECISION

Purpose: Automatically repeat homing and fixed-distance moves to evaluate homing repeatability error.

| Parameter | Description |
| --- | --- |
| axis | X, Y, Z, or R. |
| test_pulse_count | Positive pulse count moved in each test. |
| test_count | Number of tests, 1-100. |

Execution: Home the selected axis -> move the specified pulses -> repeat for the specified count -> record or report the results.

Example  TEST_PRECISION  axis=X, pulses=10000, count=10

Note: This test physically drives the axis. Ensure the entire travel path is safe and reduce the speed for the first test.

# 4 Vision Offset Instructions

### 16. Request Vision Offset  -  REQUEST_ROI_OFFSET

Purpose: Request a position offset for a specified ROI from the host vision system.

| Parameter | Description |
| --- | --- |
| roi_id | 1=Pick Offset; 2=Place Offset. |

Execution: Send the request -> wait on the current row -> advance after a valid offset is received and stored.

Example  REQUEST_ROI_OFFSET  roi_id=1

Note: If the vision system does not respond, the task remains on the current row. Use current-row highlighting to locate this wait during debugging.

### 17. Offset Compensation  -  OFFSET_COMPENSATION

Purpose: Add one stored ROI single-axis offset to the current position and move.

| Parameter | Description |
| --- | --- |
| roi_id | Offset source: 1 or 2. |
| axis | Compensate one of X, Y, Z, or R. |

Execution: Read and consume the selected-axis offset -> current axis position + offset -> check range -> execute motion and wait.

Example  OFFSET_COMPENSATION  roi_id=1, axis=X

Note: A missing offset, ROI mismatch, or out-of-range compensated target causes an error. To compensate both X and Y, use MOVE_XYZ_WITH_OFFSET or prepare the corresponding offsets separately.

# 5 IO and Timing Instructions

### 18. Write Output IO  -  IO_WRITE

Purpose: Set one Y output immediately.

| Parameter | Description |
| --- | --- |
| io_index | Y0-Y23. |
| io_value | 0=Close/ON; 1=Open/OFF. |

Execution: Set the physical output and update the internal IO state, then advance immediately.

Example  IO_WRITE  Y3=Close (0)

Note: Task completion does not restore this output automatically. Add another IO_WRITE when a reset state is required.

### 19. Delay (Milliseconds)  -  DELAY_MS

Purpose: Wait for a specified time without blocking the scheduling of other tasks.

| Parameter | Description |
| --- | --- |
| delay_ms | Wait time in milliseconds; default 100. |

Execution: Record the deadline and yield scheduling. Advance when the time expires. A value of 0 ms advances immediately.

Example  DELAY_MS  delay_ms=500

Note: This is a nonblocking wait at task level; it does not stop the entire controller.

# 6 Variables and Flow Control

### 20. Set Variable  -  SET_VAR

Purpose: Assign, increment, or decrement global variables V0-V19.

| Parameter | Description |
| --- | --- |
| variable_index | V0-V19; the special system_count item is used only for accumulated system count. |
| assign_mode | = assign, + add, - subtract. |
| value | Unsigned value. |

Execution: Perform the operation and advance immediately. Subtraction is saturated: if the current value is less than or equal to the subtrahend, the result is 0 and does not underflow.

Example  SET_VAR  V0 + 1

Note: Variables are not cleared automatically by END or task restart. When an initial value is required, explicitly place V0=0 at row 0.

### 21. Jump  -  JUMP

Purpose: Jump unconditionally to a target row in the task.

| Parameter | Description |
| --- | --- |
| target_row | Target task row, selected by the row number displayed and stored in the task table. |

Execution: Verify that the target row exists -> set the program counter to the target row -> do not execute the original next row.

Example  JUMP  target_row=2

Note: An unconditional backward jump creates a loop. Design an exit condition or terminate it with a Stop control.

### 22. Conditional Jump  -  JUMP_IF_INPUT

Purpose: Compare two input sources or values and jump when the condition is true.

| Parameter | Description |
| --- | --- |
| value 1 | Constant 0/1, X0-X39, or V0-V19. |
| condition | =, <>, <, >, <=, or >=. |
| value 2 | X state Close/Open, another X/V/constant, or a manually entered value. |
| target_row | Target row when true; Current Row is also available. |

Execution: Read and compare the two values. If true, jump to the target; otherwise, advance. Current Row keeps execution on this row while true and is useful for waiting for a condition to change.

Example  JUMP_IF_INPUT  X0 = Open  -> Current Row

Note: For X status, Close corresponds to logic 1 and Open to logic 0. When using Current Row as a wait, reversing the condition can hold the task indefinitely.

# 7 Auxiliary Motors and Indexer

Auxiliary motors are managed separately from the main XYZR interpolation motors. In multi-axis interpolation mode, the interface normally allows M4-M7 only; non-interpolation mode allows M0-M7.

### 23. Run Motor  -  RUN_MOTOR

Purpose: Start a selected auxiliary motor in continuous or fixed-pulse mode.

| Parameter | Description |
| --- | --- |
| motor_index | M0-M7; the available range depends on the operating mode. |
| run_mode | continuous or fixed pulses. |
| pulse_count | Must be greater than 0 in fixed-pulse mode. |

Execution: Continuous mode advances immediately after start and the motor keeps running. Fixed-pulse mode waits until all pulses finish before advancing.

Example  RUN_MOTOR  M4, fixed pulses=24000

Note: Continuous mode must be paired with STOP_MOTOR or an appropriate task-control strategy. The motor does not stop automatically before the next row.

### 24. Set Motor Speed  -  SET_MOTOR_SPEED

Purpose: Set the pulse frequency of a selected auxiliary motor.

| Parameter | Description |
| --- | --- |
| motor_index | M0-M7, subject to operating-mode restrictions. |
| speed_hz | Target frequency in Hz; interface default is 4000 Hz. |

Execution: Acquire the motor resource -> update the frequency -> release the temporary resource -> advance immediately.

Example  SET_MOTOR_SPEED  M4=4000 Hz

Note: The speed must be within the motor configuration limits. Set the speed before RUN_MOTOR.

### 25. Stop Motor  -  STOP_MOTOR

Purpose: Stop a selected auxiliary motor and release its task resource.

| Parameter | Description |
| --- | --- |
| motor_index | M0-M7 motor to stop. |

Execution: Send Stop -> release the corresponding motor resource -> advance.

Example  STOP_MOTOR  M4

Note: Select the same motor number used by RUN_MOTOR.

### 26. Run Indexer  -  RUN_INDEXER

Purpose: Start a motor or Y output to drive an indexer and wait for one positioning-sensor cycle.

| Parameter | Description |
| --- | --- |
| target | Auxiliary motor M0-M7 or output Y0-Y23. |
| input_index | Positioning sensor X0-X39. |

Execution: Start the target (run motor or close Y) -> if X is currently closed, wait for it to open -> wait for X to close again -> stop the motor or open Y -> advance.

Example  RUN_INDEXER  target=M4, sensor=X2

Note: It recognizes a sensor cycle of Closed -> Open -> Closed again. Incorrect sensor polarity or mounting position can leave the task waiting indefinitely.

# 8 End Instruction

### 27. End  -  END

Purpose: Mark the current task as complete and stop executing subsequent rows.

Execution: Set the task state to DONE and release the motion, motor, and IO resources owned by the task.

Example  END

Note: END does not clear V0-V19 and does not restore every Y output to an initial state. Explicitly reset required variables and outputs before END.

# 9 Complete Task Examples

## 9.1 Flash an Output Three Times

| Row | Instruction |
| --- | --- |
| 0 | SET_VAR V0 = 0 |
| 1 | IO_WRITE Y0 = Close |
| 2 | DELAY_MS 300 |
| 3 | IO_WRITE Y0 = Open |
| 4 | DELAY_MS 300 |
| 5 | SET_VAR V0 + 1 |
| 6 | JUMP_IF_INPUT V0 < 3 -> row 1 |
| 7 | END |

Explanation: Row 0 explicitly initializes V0 so that a value left by a previous run cannot affect this loop.

## 9.2 Wait for a Sensor, Then Pick and Place

| Row | Instruction |
| --- | --- |
| 0 | JUMP_IF_INPUT X0 = Open -> Current Row |
| 1 | PICK point=1, safe_z=20000, Y3=Close, 150 ms |
| 2 | PLACE point=2, safe_z=20000, Y3=Open, 150 ms |
| 3 | END |

Explanation: While X0 is open, the condition is true and execution remains on the current row. When X0 closes, the condition becomes false and the task advances to Pick. During debugging, row 0 remains highlighted and refreshes approximately every 300 ms.

## 9.3 Pick Using a Vision Offset

| Row | Instruction |
| --- | --- |
| 0 | REQUEST_ROI_OFFSET roi=1 |
| 1 | MOVE_XYZ_WITH_OFFSET point=5, roi=1, option=XY |
| 2 | IO_WRITE Y3=Close |
| 3 | DELAY_MS 150 |
| 4 | MOVE_AXIS_ABSOLUTE Z=20000 |
| 5 | END |

Explanation: Row 0 waits for vision data. Row 1 reads and consumes the X/Y offset for ROI 1, then adds it to Point 5.

# 10 Debugging, Pause, and Stop

| Function | Relationship to Program Execution |
| --- | --- |
| END instruction | Normal program completion. Variables are not cleared and outputs are not reset as a group. |
| Immediate Stop | Requests the current task to stop and end immediately. Motion, motor, and other resources owned by the task are stopped or released. |
| Buffered Stop | Waits for the current instruction to finish, then stops the task. Use it when the action should not be terminated midway. |
| Immediate Pause | Pauses at the current execution position. Firmware saves resumable state and resumes from the original position when possible. |
| Buffered Pause | Pauses after the current instruction finishes. |
| Current Row | While running or paused, refreshes and highlights approximately every 300 ms to show whether execution is in motion, waiting for input, awaiting vision, or blocked by a resource interlock. |

Debugging Recommendation First use Step Run to verify every motion target and IO polarity, then run continuously. If a task remains on one row for a long time, check that row's completion condition, sensor status, vision response, and resource ownership first.

## Common Troubleshooting

The task remains on one row after starting: check whether the instruction is a waiting type, and verify its input, vision response, motion-complete signal, or resource availability.

Variables still contain values after END: this is by design. Initialize them with SET_VAR at the beginning of the task.

Outputs remain active after END: add IO_WRITE before END to restore critical outputs to a safe state.

The indexer does not stop: confirm that the sensor completes a Closed -> Open -> Closed cycle and that the X-input polarity is correct.

An offset move reports an error: confirm that the correct ROI was requested, its offset has not already been consumed, and the resulting target is within range.

A fixed-pulse motor does not advance to the next row: it waits for every pulse to complete. Check motor enable, frequency, and pulse count.

# Appendix A - Complete Instruction Quick Reference

| Category | Instruction Code | Execution Summary |
| --- | --- | --- |
| Motion | MOVE_POINT_ID | Move to a stored point and wait |
| Motion | MOVE_XYZR | Absolute XYZR move and wait |
| Motion | MOVE_XYZ_WITH_OFFSET | Stored point plus vision offset; wait |
| Motion | MOVE_RELATIVE | Coordinated relative move and wait |
| Motion | MOVE_AXIS_ABSOLUTE | Single-axis absolute move and wait |
| Motion | MOVE_AXIS_RELATIVE | Single-axis relative move and wait |
| Interpolation | ARC_XY | XY arc interpolation and wait |
| Interpolation | ELLIPSE_XY | XY ellipse interpolation and wait |
| Process | PICK_PLACE | Complete pick-and-place sequence |
| Process | PICK | Pick and return to safe height |
| Process | PLACE | Place and return to safe height |
| Homing | HOME | Multi-axis three-stage homing |
| Homing | RESET_AXIS | Single-axis single-stage reset |
| Settings | SET_SPEED_HZ | Update motion speed and acceleration |
| Test | TEST_PRECISION | Homing repeatability test |
| Vision | REQUEST_ROI_OFFSET | Request and wait for ROI offset |
| Vision | OFFSET_COMPENSATION | Consume one-axis offset and move |
| IO | IO_WRITE | Set Y output and advance immediately |
| Timing | DELAY_MS | Nonblocking task delay |
| Variable | SET_VAR | Assign, add, or saturated subtract |
| Flow | JUMP | Unconditional jump |
| Flow | JUMP_IF_INPUT | Compare and conditionally jump |
| Motor | RUN_MOTOR | Continuous start or fixed-pulse run |
| Motor | SET_MOTOR_SPEED | Set auxiliary-motor frequency |
| Motor | STOP_MOTOR | Stop auxiliary motor |
| Indexer | RUN_INDEXER | Run until sensor completes a cycle |
| Flow | END | Complete task without clearing variables |